Meeting on AdV benches and labs

23 April 2009

Bradaschia, Calloni, Fiori, Genin, Marque, Paoli, Pasqualetti, Ruggi, Tournefier, Vajente

Agenda:

Preliminary evaluation of

1) vacuum isolation of external benches

2) modifications of labs infrastructure

Discussion:

1) About isolation options of benches

Acoustic isolation at low frequency (<50Hz), which would necessary require an in-vacuum solution, seems mandatory only in case benches need an active suspension (100mHz cutoff) to mitigate microseism. It is important to determine, by means of noise projections and yearly microseism statistics which is the impact of microseism up-conversion noise on AdV and V+MS. A first evaluation was given in eLogs 21658 and 22139 for the terminal benches. The indication is that the noise from terminal benches can be critical already (and especially) for V+MS. We would need a factor approx. 5 reduction to deal with high microseism (that is 1 to 10% of time).

It is proposed to consider the use of a phase shifter, like the one under study by GEO-ILIAS, which would reduce recombination of back-scattering light.

It is discussed the option of reducing transmission of V+MS mirrors. A factor 2-3 reduction (from present design of 10ppm to 4ppm) seems not to compromise ITF operation. There is not much time to decide, since new end mirrors coating is going to start shortly in Lyon.

The external detection bench looks presently critical for diffused light for V+MS, but projection is yet very uncertain, since we have some confidence that there is margin to further reduce diffused light fraction or coupling on EDB.

If microseism mitigation turns out not to be necessary (it is the case for EIB) increasing bench rigidity seems an option. It is useful to understand if making the benches more rigid, by how much we worsen the bench seismic noise at high frequency. Measurements of ground vs bench noise are needed.

Another option is a bench suspension with a cutoff around 1Hz, actively controlled. In this case the need of acoustic isolation with vacuum has to be better evaluated.

Details about vacuum design seems too premature, however Antonio assures that for a light vacuum (1mbar) out-gassing is not a concern, thus no heavy requirements on components are necessary.

2) About infrastructures LL

Enclosures would very likely need to be modified to host new INJ benches for AdV. It is useful to evaluate if we can actually get rid of the enclosures, while improving the isolation of the walls. This seems reasonable if we manage to relocate the MasterLaser PS rack (possible if GEO laser is adopted) and thus no more sources of noise remain inside LL.

We indeed have the suspect that already now, the major contribution to LL acoustic noise comes from the central hall. It is useful to better prove this assertion with measurements. Suggestion is to compare microphone data with enclosure close and open.

3) About infrastructures DET

Questions remain open and need further discussion also with Edwige:

- do we need to make lab a clean room?

(this would necessitate a dedicated air conditioning for overpressure / laminar air circulation).

- do we need to enlarge volume?

(this would likely require removal of the wall behind the bench, which is a concrete wall and necessitates structural modifications involving heavy and dirty works. Possible, but needs strong motivation).