| case | details | Load (mbar.l/s) | pressure at trap exit (mbar) | notes |
| --- | --- | --- | --- | --- |
| Input/end tower restart | Open LV after 4 days | 2E-4 | 2E-10 | All load goes in one trap |
| As above | Open after two days (as now) | 4E-4 | 4E-10 |  |
| DT/IB tower restart | Open LV after 4 days | 1E-3 | 4E-10 | Load divided in the two traps , 5E-4 each |
| As above | After 2 days | 2E-3 | 1E-9 | extrapolated |
| Long term operation: 5 uhv towers +2 bench towers | After months | 3E-4 | 9E-11 | Run conditions (Load is splitted between two traps) |

Tab. x Water loads for traps evaluation